Appendix 3 / September 9, 2002

STRUCTURAL AND EQUIPMENT REQUIREMENTS FOR BUSES USED IN TENDERED SERVICES

New buses purchased for tendered services must be either semi-low-floor or low-floor vehicles, of which either the front section or the whole body has to be of low-floor construction. The buses already in use or purchased second-hand must meet the YTV requirements for bus fleet valid at the time of purchase.

In addition to the structural provisions, buses purchased for tendered services must meet the following structural and equipment requirements:

- Vehicle identification

The bus operator’s name has to appear on both sides of the bus in letters at least 80 mm high. The serial number of the bus must be shown on both sides and at the rear of the bus in digits at least 100 mm high, and inside the bus in digits c. 30 mm high. The regional transport symbol must be displayed at the front of the bus.

- Boarding the bus

The boarding height must not exceed 350 mm at any door. The step height must not exceed 250 mm even at the back door. The bus must be equipped with a full-length kneeling facility.

- Seat height

The front edge of each passenger seat must be approximately 450 mm high, except for possible elevated seats for disabled passengers and seats over wheel arches.

- Wheelchair ramp

Each low-floor bus must have a wheelchair ramp at the middle door. If the ramp is controlled by the driver, it must have safety devices.

- Space for prams/wheelchairs

Each bus must have a space for at least two prams. The space has to be outfitted also for wheelchairs. The push buttons must be placed on vertical grabs bars on both sides of the aisle at the center section, and on the wall of the pram space as well as on the outside to the right of the middle door. The seats must be signposted. Tip-up seats may be used in the pram space.

- Seats for disabled passengers

Each bus must have four seats reserved for disabled passengers in front of the middle door (the seats are to be located on both sides of the aisle, i.e. 2+2). One seat for a disabled passenger with a guide dog must be located in the first row behind the driver. The seat next to it must be a tip-up seat, unless provided with a legroom of at least 450 mm. The seats must be signposted.

- Push buttons

The push buttons must be placed on the grab bars at the maximum height of 1, 550 mm. In addition, there must be a push button for children next to exit doors at the maximum height of 1, 000 mm. The push buttons for disabled passengers must be placed so that from each seat for disabled passengers the stop signal can be given without having to stand. The colors of the push buttons have to be chosen bearing in mind the weak-sighted. Pressing of the push buttons must produce an audible signal.

- Grab bars and grips

In order to make traveling easier for the mobility impaired, a grip must be installed on the wall next to the seats meant for these passengers, unless the seat in front is equipped with a suitable grip or other aid. Also both horizontal and vertical grab bars should be placed so that it is possible to move from the entrance to the exit door using the grab bars and grips. If the seats for disabled passengers are placed so that there is no seat in front of them, grab bars must be provided in front of them. The doors must be equipped with grips suitable for mobility impaired passengers.

9.   Door interlock

Each bus is to be equipped with a door interlock mechanism which prevents driving with the doors open (incl. the front door). The door interlock must be on when the vehicle is in service.

- Terminus equipment

Each bus has to be equipped with terminus equipment enabling the passengers to get on or off the bus at the terminus via the front and middle doors even when the driver is not on board. Thus, starting the vehicle and its unauthorized seizure must be prevented.

- Door safety devices

All doors must be equipped with safety devices.

- Illuminated and informative signs

An illuminated stop sign (‘Pysähtyy/Stannar’ or ‘STOP’) must be located in a conspicuous place at the front part of the bus, in articulated buses there should be a sign also behind the articulated joint.

An illuminated FULL sign (‘Täynnä/Fullsatt’) has to be placed at the upper corner of the windshield. Any possible permanent written passenger information signs inside the vehicle must be bilingual (Finnish-Swedish).

- Route number displays

Each bus must have an illuminated and clearly visible route number display at the front and rear and on the right-hand side. The route number at the front must be at least 280 mm high, and the numbers at the rear and on the right-hand side at least 160 mm. The height of letters must be 50 to 80 per cent of the height of the numbers. Any texts written on the route number displays must be clearly legible. The driver must be able to change the display from their seat, and a destination board with a minimum height of 70 mm must be placed also inside the bus.

- Other boards

If there is need for an additional service number board, it should be placed in the side window next to the front door.

- Information material

The holder for timetables and brochures have to be placed at the front of the bus so that it is accessible to both the passengers and the driver. The holders must be large enough for A5 – sized material.

- Fare collection equipment and other on-board equipment

The fare collection equipment and other equipment required by the client must be placed according to the client’s instructions. New vehicles purchased must be adapted for installing the known equipment.

- Space for information inside the bus

The area on top of the side windows must be reserved for the public transport fare, instruction and advertisement stickers. The area should be 250 mm high and 1, 200 mm in length, and it must be smooth and without transversal cleats. Moreover, there must be space to attach public transport bulletins by the standing area or exit doors.

- Interior noise level

The noise level in the bus, measured in accordance with the decision of the Ministry of Transport and Communications, must not exceed the level mentioned in the decree even at the rear of the bus (78 dBA).

- Ventilation

The intake must be located above the level of the side windows and the intake pipe has to be equipped with a filter. The vehicles must be equipped with ceiling duct fans.

- Interior lightning

Each bus must have interior lightning enabling all seated passengers to read normal sized text (100 – 200 lux) with ease.

- Exit door lights

The exit doors must be equipped with lights which illuminate also the area outside the exit door.

- Clock

There must be a clock visible to the passengers in each bus.

VEHICLE TYPES AND SPECIFICATIONS, NEW VEHICLES

TYPE A

- Low-floor buses, either with full-length low floor or with low-floor front section.

- Buses with low-floor front sections must not have any steps between the front and middle doors. Full-length low-floor buses must be completely stepless.

- Doors: minimum 2+2+1.

- Minimum seating capacity (fixed seats) 34, and total seating capacity 65 minimum.

TYPE B

- So called semi low-floor buses in which the floor level is max. one step higher than in low-floor buses.

- The vehicles must have stepless aisles.

- Doors: minimum 2+2+0.

- Minimum seating capacity (fixed seats) 38, and total seating capacity 65 minimum.

TYPE C (BOGIE BUSES)

- Low-floor buses either with full-length low floor or with low-floor front section.

- Buses with low-floor front sections must not have any steps between the front and middle doors. Full-length low-floor buses must be completely stepless.

- Doors: minimum 2+2+2.

- Minimum seating capacity (fixed seats) 51, and total seating capacity 87 minimum.

TYPE D (ARTICULATED BUSES)

- Low-floor buses either with full-length low floor or with low-floor front section. Buses with full-length low floor must be completely stepless

- Doors: 2+2+2.

- Minimum seating capacity (fixed seats) 52, and total seating capacity 100 minimum.

CLEARANCE MEASUREMENTS COMMON TO ALL BUS TYPES

- For one-section doors, minimum clear opening 750 mm.

- For two-section doors, minimum clear opening 1, 200 mm.

- Minimum aisle width 550 mm.

- Minimum unobstructed longitudinal seating space 710 mm.

- Minimum unobstructed length of the pram space 1, 300 mm.

MISCELLANEOUS

- With the consent of the client, so-called country buses are allowed on occasional rush-hour departures.

- The medium age of the bus fleet and the age of an individual bus are calculated in years from the first registration date, to an accuracy of one decimal point.

FLEET REQUIREMENTS FOR SMALL BUSES (MINI)

- cabin with full-length low floor or low-floor front section

- minimum standing height in the cabin 1, 900 mm

- passenger doors at least 1+0 or 2+0

- unobstructed doorway at least 750 mm

- vehicle length approximately 7,0 m

- vehicle width 1,9 – 2,3 m

- space for a pram/wheelchair, wheelchair tie-downs available in the wheelchair space

- minimum seating capacity (including tip-up seats) 13

FLEET REQUIREMENTS FOR SMALL BUSES (MIDI)

- cabin with full-length low floor or low-floor front section

- minimum standing height in the cabin (including standing room and the space for prams) 1, 900 mm

- passenger doors at least 1+1

- unobstructed doorway at least 750 mm

- vehicle length approximately 8,5 m

- vehicle width over 2,2 m

- space for two (2) prams / one wheelchair, wheelchair tie-downs available in the wheelchair space

- minimum seating capacity (including tip-up seats) 20

STRUCTURAL AND EQUIPMENT REQUIREMENTS FOR SMALL BUSES (MINI)

In addition to the structural requirements for buses, the vehicles purchased for the tendered services have to meet the following structural and equipment requirements:

- Vehicle identification

The bus operator’s name has to appear on both sides of the bus in letters at least 80 mm high. The serial number of the bus must be shown on both sides and at the rear of the bus in digits at least 100 mm high, and inside the bus in digits c. 30 mm high. The regional transport symbol must be displayed at the front of the bus.

2. Boarding the bus

The boarding height must not exceed 350 mm at any door. The step height must not exceed 250 mm even at the back door.

3. Seat height

The front edge of each passenger seat must be approximately 450 mm high, except for possible elevated seats for disabled passengers and seats over wheel arches.

4. Grab bars and grips

In order to make traveling easier for the mobility impaired, a grip must be installed on the wall next to the seats meant for these passengers, unless the seat in front is equipped with a suitable grip or other aid. Also both horizontal and vertical grab bars should be placed so that it is possible to move from the entrance to the exit door using the grab bars and grips. If the seats for disabled passengers are placed so that there is no seat in front of them, grab bars must be provided in front of them. The doors must be equipped with grips suitable for mobility impaired passengers.

5. Route number displays

Each bus must have a clearly visible route number display at the front. If there is need for an additional service number board or other route identifier, it should be placed in the side window next to the front door.

6. Information material

The holder for timetables and brochures have to be placed at the front of the bus so that it is accessible to both the passengers and the driver. The holders must be large enough for A5 – sized material.

7. Fare collection equipment and other on-board equipment

The fare collection equipment and other equipment required by the client must be placed according to the client’s instructions. New vehicles purchased must be adapted for installing the known equipment.

- Interior lightning

Each bus must have interior lightning enabling all seated passengers to read normal sized text (100 – 200 lux) with ease.

- Age

The average age of the bus fleet and the age of an individual bus are calculated in years from the firs registration date, to an accuracy of one decimal point

- Clock

There must be a clock visible to the passengers in each bus.

STRUCTURAL AND EQUIPMENT REQUIREMENTS FOR SMALL BUSES (MIDI)

In addition to the structural requirements for buses, the vehicles purchased for the tendered services have to meet the following structural and equipment requirements:

1.Vehicle identification

The bus operator’s name has to appear on both sides of the bus in letters at least 80 mm high. The serial number of the bus must be shown on both sides and at the rear of the bus in digits at least 100 mm high, and inside the bus in digits c. 30 mm high. The regional transport symbol must be displayed at the front of the bus.

2. Boarding the bus

The boarding height must not exceed 350 mm at any door. The step height must not exceed 250 mm even at the back door.

3. Seat height

The front edge of each passenger seat must be approximately 450 mm high, except for possible elevated seats for disabled passengers and seats over wheel arches.

- Wheelchair ramp

Each low-floor bus must have a wheelchair ramp at the middle door. If the ramp is controlled by the driver, it must have safety devices.

- Space for prams / wheelchair

Each bus must have a space for at least two prams. The space has to be outfitted also for wheelchairs; wheelchair tie-downs must be available in the wheelchair space.

The push buttons must be placed on vertical grabs on both sides of the aisle at the center section, and on the wall of the pram space as well as on the outside to the right of the middle door. The seats must be signposted. Tip-up seats may be used in the pram space.

- Seats for disabled passengers

Each bus must have two seats reserved for disabled passengers on the front of the middle door. The seats must be signposted.

- Push buttons

The push buttons must be placed so that from each seat the stop signal can be given without having to stand. The colors of the push buttons have to be chosen bearing in mind the weak-sighted. Pressing of the push buttons must produce an audible signal.

- Grab bars and grips

In order to make traveling easier for the mobility impaired, a grip must be installed on the wall next to the seats meant for these passengers, unless the seat in front is equipped with a suitable grip or other aid. Also both horizontal and vertical grab bars should be placed so that it is possible to move from the entrance to the exit door using the grab bars and grips. The doors must be equipped with grips suitable for mobility impaired passengers.

- Door safety devices

All doors must be equipped with safety devices.

10. Illuminated signs and informative signs

An illuminated stop sign (‘Pysähtyy/Stannar’ or ‘STOP’) must be located in a conspicuous place at the front part of the bus.

An illuminated FULL sign (‘Täynnä/Fullsatt’) has to be placed at the upper corner of the windshield. Any possible permanent written passenger information signs inside the vehicle must be bilingual (Finnish-Swedish).

11. Route number display

Each bus must have an illuminated and clearly visible route number display at the front and rear and on the right-hand side.

12.Other boards

If there is need for an additional service number board, it should be placed in the side window next to the front door.

13. Information material

The holder for timetables and brochures have to be placed at the front of the bus so that it is accessible to both the passengers and the driver. The holders must be large enough for A5 – sized material.

14. Fare collection equipment and other on-board equipment

The fare collection equipment and other equipment required by the client must be placed according to the client’s instructions. New vehicles purchased must be adapted for installing the known equipment.

15. Space for information inside the bus

The area on top of the side windows must be reserved for the public transport fare, instruction and advertisement stickers. The area should be 250 mm high and 1, 200 mm in length, and it must be smooth and without transversal cleats. Moreover, there must be space to attach public transport bulletins by the standing area or exit doors.

- Interior noise level

The noise level in the bus, measured in accordance with the decision of the Ministry of Transport and Communications, must not exceed the level mentioned in the decree even at the rear of the bus (78 dBA).

- Ventilation

The intake must be located above the level of the side windows and the intake pipe has to be equipped with a filter. The vehicles must be equipped with ceiling duct fans.

- Interior lightning

Each bus must have interior lightning enabling all seated passengers to read normal sized text (100 – 200 lux) with ease.

- Exit door lights

The exit doors must be equipped with lights which illuminate also the area outside the exit door.

- Age

The average age of the bus fleet and the age of an individual bus are calculated in years from the firs registration date, to an accuracy of one decimal point.

21. There must be a clock visible to passengers in each bus.